Name: _______________________________

HWR 201

Assignment 3: Hydraulics

PART 1: Answer the following questions from the readings.

- What is pressure?

- What is hydrostatic pressure?

- List four important characteristics of hydrostatic pressure.

- Use Figure 2.1 below.  If the pressure at Point A is 49kPa and the pressure at point B is 88 kPa, what is the pressure at Point C?  Explain.

- What is pressure head?

- What is discharge?  Give a few examples of units for discharge.

- Explain the Continuity Equation and the principle of Continuity of Flow.

- Imagine water flowing through a pipe.  If you took an imaginary slice of the pipe, would the velocity of water at the center of the pipe equal the velocity of water just barely touching the pipe on the side?  Explain.

- What is steady uniform flow?

- What is Manning’s Equation used for?

- What is a wetted perimeter?

- What does the “n” in Manning’s Equation represent?

PART 2:  Problem Set

- Refer to the picture of a water tower and house below.  Calculate the hydrostatic pressure at Points A, B, C, D, and E.   See Example 2.2 and Figure 2.1 in Nathanson (and above). Hint: remember that the pressure depends on the depth of water (or equivalent amount of water) above the point.

- Use Q = A*V for the following problems. See Example 2.7 in Nathanson.

  - If a small stream has a velocity of 5 ft/s and a cross-sectional area of 120 ft2, what is the discharge in ft3/s?

  - If a river has a discharge of 75,000 ft3/s and a cross-sectional area of 8,000 ft2, what is the velocity of the river in ft/s?

- Find the peak discharge, Q, using Manning’s Equation if the stream slope is 0.0002 and it is a natural channel that is sluggish, weedy, and with deep pools.  The dimensions of the channel are shown below.  (See example handout and use Figure 5 in streamflow handout to determine n.)

- Find the peak discharge, Q, using Manning’s Equation if the stream slope is 0.0015 and the stream is lined with ordinary concrete (like some washes).  The dimensions of the channel are shown below.  The depth of the water is 27 ft.

**Hint: Area of a trapezoid =  )