Name: _______________________________

HWR 201

Assignment 8: Surface Water Pollution

PART 1: Answer the following questions.

- Explain what it means for water to be “polluted”.

- What is the difference between point source and nonpoint source pollution?  Which is easiest to prevent and/or clean-up?

- Describe the effects of thermal pollution in a river or stream.

- Describe the effects of sediment pollution.

- Describe how the release of untreated sewage into waterways can cause problems.

- Explain the factors that affect a stream’s ability to self-purify.

- Describe and explain the four zones of pollution in a waterway from when biodegradable organic substances are released into it and until the stream recovers.

- What does an oxygen sag curve tell you?

- What is meant by the critical oxygen deficit?

PART 2: Problem Set

Mass Balance Equation

- An effluent from a sewage treatment plant has a TDS concentration of 300 mg/L and a discharge of 35 mgd (million gallons/day). The receiving stream has a TDS of 90 mg/1 and a discharge of 150 mgd. Compute the TDS concentration in the combined sewage and streamflow downstream of the mixing zone (the diluted concentration). Assume complete mixing.

- The BOD5 of an effluent from a poorly operating sewage treatment plant is 400 mg/L and the discharge is 20 ML/d, (megaliter/day).  The receiving stream has a BOD5 of 10 mg/L.  What minimum flow in the receiving stream is needed for a dilution such that the combines BOD5 of the mixed water is no greater than 20 mg/L?

- The combined BOD5 in a stream mixed with sewage effluent is 25 mg/L.  The BOD5 of the stream is originally 8 mg/L and its flow before mixing is 200 mgd.  If the combined flow is 250 mgd, what is the BOD5 of the sewage effluent?