Name: _______________________________

HWR 201

Assignment 10: Stormwater Management

PART 1: Answer the following questions.

- Stormwater management is both a _q____________ and __q____________ problem.  Explain.

- Define time of concentration of a drainage basin.  How is it determined and what is it used for?

- What is a runoff coefficient? What is it used for?

- Thinking about the Rational Formula, would you expect to have a greater peak runoff rate over an area where C = 0.25 or an area where C = 0.70?

- What is a runoff curve number?  How does it differ from the runoff coefficient used in the Rational Formula?

- Give two direct effects of urbanization on stormwater runoff.  What is the overall effect on a short-term basis and in the long run?

- Briefly describe three basic differences between storm sewer systems and sanitary sewer systems.

- What are BMPs?  Describe the two basic BMPs used in stormwater management.

- List five benefits of on-site stormwater storage.

- What are some disadvantages of stormwater storage basins?

- Briefly describe 3 different types of stormwater storage basins.

- What is a floodplain, flood hazard area, and flood fringe?

- What happens to stormwater runoff if it is collected in a combined sewer system?

See next page for Part 2: Problem Set !!!!!

PART 2: Problem Set

Rational Formula

where   Q = peak or maximum rate of runoff (ft3/s)

C = is a dimensionless runoff coefficient

i = rainfall intensity (in/hr)

A = drainage basin area (acres)

Note:  Even though when you multiply the units of in/hr and acres, the answer, Q,  is in ft3/s.  You don’t have to do any conversions.  See Example 9.1 in Nathanson)

- A) Using the Rational Formula, estimate the peak rate of runoff on a 3.5 acre watershed from a storm with a rainfall intensity of  i = 2.0 in/hr. Assume C = 0.70.

- What type of land surface is this watershed? (Use Table 9.1)

- What is the peak runoff rate in m3/hr? (Use your answer from A)

- What is the peak runoff rate in acre-feet/day? (Start with your answer from A)

- A) Using the rainfall frequency diagram (Fig 3.5), determine the rainfall intensity in (in/hr) for a storm duration of 30 min. and a return period of 25 years.

B) From that intensity, use Figure 9.5 to determine the number of inches of runoff that can be expected to fall onto that watershed in one hour if the curve number, CN is 70. (Hint: Intensity = inches/ one hour)

C)  If the watershed area is 500 acres, what is the volume of runoff (in ft3) due to the storm? (Hint: you must convert units).