Name: _______________________________

HWR 201

Assignment 4: Hydrology

PART 1: Answer the following questions from the readings.

- On a separate piece of paper, draw a picture depicting the water cycle.  Include and label the following: solar energy, evaporation, condensation, precipitation, infiltration, evapotranspiration, surface water (river, lake, or ocean), groundwater, surface runoff, water table.

- Review the Water Cycle Handout and fill in the blanks.

About 97% of the earth’s water is _______   _________ located in the oceans.  About _____% of the earth’s water is freshwater.   Of that freshwater, about 2% is located in _______  ________ and  ____________.  Therefore, less than  _____ % of the earth’s water is freshwater that the people of the earth can utilize to sustain their lives.

- What is rainfall intensity?

- What is a recurrence interval (or) return interval?

- What is meant by an n-year storm?

- How does the recurrence interval relate to the probability of a certain size storm occurring?

- What is a watershed (or drainage basin)?  What is the boundary of a watershed called?

- What is a hydrograph?

- What is baseflow of a stream?

- What is stream stage?

- What is a rating curve and why is it useful?

- What are some of the many purposes of water reservoirs?

PART 2:  Problem Set

- See Example 3.1 in Nathanson.

A. During a 30 minute rain storm, the average rainfall depth over a 500 m2 watershed was 40 inches.  How much volume does that represent in meters3? (Hint: Volume = Area * height)

B.  If 20% of that volume runs off, how much volume is that, in gallons?

C. What was the rainfall intensity?

- The average annual rainfall over a 200 mi2 watershed is 35 inches.  How much volume does that represent in acre-ft?

- Using the Rainfall Curve in Figure 3.5 in your textbook and attached:

- Determine the rainfall intensity of a 100 year storm with a rainfall duration of 80 min.

- Determine the rainfall intensity of a 25 year storm with a rainfall duration of 35 minutes.

- Determine the rainfall duration of a 5 year storm with an intensity of 125 mm/hr.

- Determine the n-year of a storm with an intensity of 5.5 in/hr and a duration of 15 minutes.

- Use the rating curve below to answer the following questions. **Hint: cfs means cubic feet per second or ft3/s.

Example:  What is the discharge if a stage of 60 feet is recorded at Gage 58 during a storm?

Go to the stage of 60 feet.  Follow across the 60 feet graph line until your finger hits the drawn curve. Then drop straight down and read off the discharge. Be careful writing the discharge.  For example, you would read off 100, but your answer is 100,000 cfs.

- In August 1976, a stage of 40 feet was recorded at gaging station 58 in the Patapasco River during a storm.  Assuming that the cross-sectional area of the stream has not changed since 1975, what is the estimated discharge?

- If a stage of 35 feet was recorded, what was the discharge?

- If the discharge was 70,000 cfs, what was the stage?

- If the discharge was 16,000 cfs, what was the stage?

- Again use the above rating curve.

- If a stage of 56 feet was recorded, what is the estimated discharge?

- Using your answer from part A of this question, if the cross-sectional area of the stream is 60,000 ft2, what would you estimate the average velocity to be? (Look back at Assignment 3)