Name: _______________________________

HWR 201

Assignment 9: Drinking Water Treatment

PART 1: Answer the following questions.

- Define potable water.

- Describe the primary objective of water purification.

- Which requires more extensive treatment for purification, surface or groundwater?  Why?

- What is the SDWA and what is its purpose?

- Describe the differences between primary and secondary drinking water standards.

- What does MCL stand for?

- List and describe the five general categories of contaminants that are controlled and limited under the SDWA primary drinking standards.

- Is a narrow and deep settling tank more effective at removing suspended solids than a wide shallow tank?  Explain.

- What is detention time?

- Briefly describe the process of coagulation.  How does it improve the purification of drinking water?

- What is the purpose of filtration?  What are the key characteristics of a rapid filter?

- Clarification refers to the processes of ________________, _________________, and ________________. (See Section 6.5)

- What is considered to be the most important water treatment process with respect to preventing the spread of waterborne disease?  Is there any potential harmful side effect from this process?  Explain.

- What is the difference between chlorine demand and free chlorine residual?  What is breakpoint chlorination?  What is a DPD test used for?

- Briefly describe how ion-exchange water softening works.

- Activated carbon is often used in filters not only in large water treatment plants but in kitchen faucet water filters.  What are the two important characteristics of activated carbon that help it improve water quality?  Explain how activated carbon improves the water quality.

- Explain the two basic methods used to desalinate water.

PART 2: Problem Set

Detention Time Equation

where TD = detention time

V = volume of water in tank

Q = average flow rate (volume per unit time)

Hint: Make sure you have the same volume units in V and Q (eg. Liters and liters, gallons and gallons)

- A sedimentation tank has a volume capacity of 37,000,000 liters.  If the average flow rate entering the tank is 140,000,000 liters/day, what is the detention time (in hours)?

- A sedimentation tank has a volume of 5.5 acre-ft.  If the average flow rate entering the tank is 29,500 m3/day, what is the detention time (in hours)?