HWR 201: Correspondence

FINAL EXAM

CLOSED BOOK (80 pts total)

PART A: Multiple Choice (2 pts each)

- Referring to the picture on the right, which of the following statements is true regarding hydrostatic pressure?

- The hydrostatic pressure is the same at Points A and B.

- The hydrostatic pressure is the same at Points A and C.

- The hydrostatic pressure is more at Point A than Point C.

- The hydrostatic pressure is less at Point D than Point B.

- Communicable diseases are usually transmitted by organisms called:

- Antigens

- Pathogens

- Endemics

- Colloids

- Storms with a recurrence interval of 10 years:

- Have a ten percent chance of happening in any one year.

- Will always have ten years between occurrences.

- Drop less rain than storms with a 20% chance of happening in one year.

- Are always higher in intensity than storms with a smaller recurrence interval.

- A confining layer in an artesian aquifer is most likely made of:

- Gravel

- Sand

- Silt

- Clay

- You have samples of water from two different streams.  Both samples have a pH of 6.0.  After adding 3 drops of acid to each, Sample 1 still has a pH of 6.0 but Sample 2 now has a pH of 4.5.  Which of the following can you conclude?

- Sample 1 is less polluted than Sample 2.

- Sample 1 has a greater conductivity than Sample 2.

- Sample 1 has a greater hardness than Sample 2.

- Sample 1 has a greater alkalinity than Sample 2.

- Maximum Contaminant Levels (MCLs) have been established by the EPA primarily:

- For aesthetic purposes

- To reduce water hardness

- To protect public health

- To reduce water consumption in cities and towns

- Which statement  is true with respect to the BOD (Biochemical Oxygen Demand) of water:

- The less organic material, the higher the BOD.

- The more organic material, the higher the BOD.

- Organic material does not affect the BOD.

- The higher the amount of chemical constituents, the lower the BOD

- Thermal pollution is primarily responsible for:

- Low oxygen levels, high evaporation rates, and increased metabolism in fish.

- High oxygen levels, decreased infiltration and improved conditions for most fish life.

- Very high acid levels and increased sedimentation.

- A tendency to produce colloids, reduce transparency in water, and produce poor tasting water.

- Two nutrients that contribute to the growth of algae and the eutrophication of lakes are:

- Lead and iron

- Calcium and magnesium

- Fluoride and chlorine

- Nitrogen and phosphorus

- The oxygen sag curve shows the effect of:

- Organic pollution on the DO level in a stream

- The decrease of DO in colder water

- An increase in the level of calcium and magnesium in a stream

- The addition of chlorine to water and its affect on DO

- Water that is suitable for drinking is referred to as:

- Inorganic

- Aerated

- Potable

- Flocculated

- A primary purpose for adding coagulants to water is to:

- Produce electrostatic charges on colloidal particles.

- Reduce the hardness in water by precipitating calcium

- Produce flocs in water that precipitate out of solution.

- Produce very small compounds that float for skimming.

- Even with chemical coagulation and sedimentation by gravity, all suspended particles in water are not removed.  These are frequently removed by a physical process called:

- Radiation

- Absorption

- Filtration

- Ion exchange

- Time of concentration for a drainage basin includes:

- Overland flow time and interflow time

- Channel flow time and groundwater flow time

- Channel flow time and overland flow time

- The flow time of perched water and artesian flow time

- Land development will usually:

- Reduce runoff peaks and volumes.

- Increase peak flows and increase the volume of runoff.

- Decrease flood volumes but increase peak flows,

- Produce longer return periods for a given flood flow.

- The portion of a floodplain that is inundated by the 100-year flood is called the:

- Floodway

- Flood zone

- Flood fringe

- Flood hazard area

- In the Rational Formula, the greater the runoff coefficient, C,

- The greater the slope of the watershed

- The greater the peak runoff rate

- The greater the area wetted by the storm

- The less the peak runoff rate

- In order to lower BOD to regulated levels, the minimum treatment of sewage water in the United States generally requires:

- Primary treatment

- Secondary treatment

- Tertiary treatment

- Tertiary treatment and effluent polishing

- The removal of additional BOD or Total Suspended Solids (TSS) after primary and secondary treatment has been done is called:

- Contact stablization

- Pilot plant treatment

- Effluent polishing

- Extended Aeration

- If a city attains “safe yield” with its aquifer, this means that

- The city is taking out and using only groundwater that is safe to drink as determined by the SDWA.

- The city is taking groundwater out of the aquifer at the same rate that the aquifer is being replenished by natural or artificial recharge.

- The city is allowing farms and industry to take only a minimum amount of groundwater as needed to turn a profit for their business.

- The city has a monitoring program in place to make sure that their wells are clean and functioning properly.

- What agency regulates water use in Arizona?

- NPDES

- ADEQ

- EPA

- ADWR

- Navigable streams are regulated by the Federal government.  Which statement is true concerning how streams are determined to be “navigable”?

- A stream is navigable only if it has continuously been used for navigation.

- A stream is navigable only if it can float commercial vessels such as barges, fishing boats, and cargo ships.

- A stream is navigable if a vessel of any type, including a raft or a piece of floating timber can float down it.

- A stream is navigable if it can float a small sailboat.

- With respect to Municipal Solid Waste (MSW), which statement is true?

- About one-half of the total refuse is paper

- The United States generates less waste than Japan.

- MSW generated in households is frequently hazardous

- In the United States, less than one pound of refuse is generated per day.

- Old, un-lined landfills pose a hazard to groundwater because:

- The methane gas generated by the decomposing of the waste can seep into the groundwater.

- The methane gas generated by the decomposing of the waste usually explodes underground, making a crack for groundwater to escape and be wasted.

- Leachate containing organic and metal pollutants can seep into the groundwater.

- The landfill waste can become heavy and collapse into the groundwater.

PART B: Short Answer

- List at least 3 sources of point pollution and 3 sources of non-point pollution.  (6 pts)

- List and describe the 4 zones of pollution which form when a point source of biodegradable organic substances (eg. sewage) are released into a stream.  (8 pts)

- Describe the pros and cons to using chlorine as a disinfectant in drinking water purification. (6 pts)

- What is the purpose of primary wastewater treatment?  What is the purpose of secondary wastewater treatment? (4 pts)

- List the differences between water rights assigned by riparian water law as compared to water rights assigned by prior appropriation water law?  (8 pts)